PepperDash Essentials v1.0.x Installation

Via Crestron Toolbox Text Console:

- Download PepperDash Essentials.zip from Portal

  - Open Zip file

  - Using FTP client, copy Program01 folder contents (.cpz file) to processor’s Program01 folder

- Load program without configuration using “progload -p:1” in console.  This will create the file structure on the processor

- Download any needed touchpanel files from Portal

  - Load vtz to touchpanels

  - Load iPad core3 project to processor using Toolbox

  - Load each panel type’s .sgd file to \NVRAM\Program1\sgd folder. TSW 560, 760, 1060, and/or iPad.

- Download configurationFile.json from Portal

  - Load configurationFile.json to processor’s \NVRAM\Program1 folder

- Reset program using “progreset -p:1” in console.